# MIPS Rev 6.0 — STM32H743ZIT6 Pin Planning Worksheet

CubeMX preparation companion to the MIPS Controller Hardware Upgrade Design Spec

Package: LQFP-144  |  Framework: hybrid LL + HAL

GAA Custom Electronics, LLC   |   Draft v0.1   |   July 2026

# How to Use This Worksheet

This worksheet proposes a concrete pin assignment for every MIPS peripheral function on the STM32H743ZIT6 (LQFP-144), so the CubeMX session becomes a guided confirmation rather than live problem-solving. It is organized to be worked top to bottom in CubeMX.

| ⚡ VERIFY | Every pin and alternate-function (AF) number here is a PROPOSED starting point, not gospel. The STM32H743 AF table is large and CubeMX is the authoritative validator — it enforces which peripheral signals are legal on which pins and flags conflicts live. As we enter each assignment, CubeMX will confirm it (pin turns green) or reject it (forcing a documented alternative). Treat any AF number here as “expected AFx — confirm in CubeMX.” The QSPI and timer AF mappings in particular have several valid variants across AF9/AF10 and AFs 1–3; the values here are the most common choices but must be checked. |
| --- | --- |

### Legend

- ✓ SOLID — assignment is fixed by silicon or near-universal convention (e.g., SWD pins, USB FS pins); very low risk.

- ⚡ VERIFY — a real choice among valid options; confirm the specific pin/AF in CubeMX and check for conflicts.

- LL / HAL — recommended CubeMX code-generation setting for that peripheral (per the hybrid approach).

| ℹ INFO | Framework recap (hybrid): timing-critical peripherals generate LL code (deterministic, register-precise); convenience/stack-heavy peripherals generate HAL code (mature, saves effort). Set this per-peripheral in CubeMX under Project Manager → Advanced Settings → Driver Selector. |
| --- | --- |

# Step 1 — Project Creation & Clock Tree

Before any pins, set the foundation. In CubeMX: File → New Project → select STM32H743ZITx (the ZIT6 is the LQFP-144, 2MB flash variant).

### 1.1 Clock Tree Targets

| Clock | Target | Why |
| --- | --- | --- |
| HSE (external crystal) | 25 MHz | Chosen in the BOM (Section 11.2.1) — cleanly derives both 480MHz core and exact 48MHz USB |
| SYSCLK / core | 480 MHz | Full Cortex-M7 performance (requires Rev V silicon + VOS0 voltage scaling) |
| USB clock | exactly 48 MHz | Mandatory for USB to enumerate; verify the PLL path lands on exactly 48.000 MHz |
| TIM1 kernel clock | ≥ 204.8 MHz | Required for true 12-bit PWM at 50 kHz (Design Spec §5.4.3) — set deliberately, do not accept a lower default |
| TIM2/TIM5 kernel clock | note the actual value | Feeds pulse-generator timing math — pass this exact Hz into the driver's begin() |
| Voltage scaling | VOS0 | Needed to license the 480MHz core clock on Rev V silicon |

| ⚡ VERIFY | The 48MHz-exact USB clock and the ≥204.8MHz TIM1 clock are the two clock-tree constraints most likely to be missed. CubeMX's Clock Configuration tab will highlight illegal clock rates in red; resolve both before moving on. Note the resolved APB timer clock value(s) — you will need the TIM2/TIM5 number for the pulse timer driver. |
| --- | --- |

# Step 2 — Debug, Boot & Reset (assign first)

Enable these first so CubeMX reserves the dedicated pins before you place anything else.

| Function | Pin | AF / Note | Status | Driver |
| --- | --- | --- | --- | --- |
| SWDIO | PA13 | AF0 (SYS_JTMS-SWDIO) | ✓ SOLID | — |
| SWCLK | PA14 | AF0 (SYS_JTCK-SWCLK) | ✓ SOLID | — |
| SWO (optional trace) | PB3 | AF0 (SYS_JTDO-SWO) — only if you want SWV trace; otherwise free PB3 for other use | ⚡ VERIFY | — |
| NRST | NRST (dedicated pin) | Hardware reset — not a GPIO | ✓ SOLID | — |
| BOOT0 | BOOT0 (dedicated pin) | Route to a jumper/header for ROM-bootloader recovery (Design Spec §7.1). Not a customer button. | ✓ SOLID | — |

| ℹ INFO | In CubeMX, set SYS → Debug = “Serial Wire” (this assigns PA13/PA14 as SWD and leaves the former JTAG pins PA15/PB3/PB4 free for other peripherals — which this plan uses). Do NOT select “Trace Asynchronous Sw” unless you specifically want SWO on PB3. |
| --- | --- |

# Step 3 — USB Composite Device (CDC + MSC)

Design Spec §6.1. Full-speed device, internal PHY, no external PHY parts. Generates HAL code (the ST USB composite CDC+MSC middleware is HAL-based).

| Function | Pin | AF / Note | Status | Driver |
| --- | --- | --- | --- | --- |
| USB_OTG_FS_DM | PA11 | AF10 | ✓ SOLID | HAL |
| USB_OTG_FS_DP | PA12 | AF10 | ✓ SOLID | HAL |
| USB_OTG_FS_ID | PA10 | AF10 — optional for device-only; can omit if strictly device | ⚡ VERIFY | HAL |
| VBUS sense | PA9 or a GPIO ADC | Device self-powered vs bus-powered affects whether VBUS sensing is wired; confirm | ⚡ VERIFY | HAL |

| ℹ INFO | CubeMX: enable USB_OTG_FS in “Device_Only” mode, then under Middleware enable USB_DEVICE with class = the composite/CDC+MSC option. Note: PA9/PA10 are also natural USART1 pins — if VBUS/ID take them, route the primary UART elsewhere (this plan puts the primary console UART on USART3, keeping PA9/PA10 available for USB). Watch this conflict in CubeMX. |
| --- | --- |

# Step 4 — Serial / Communications

Design Spec §5.1. Four logical UARTs, two SPI buses, two I2C buses. UART/SPI/I2C housekeeping generates HAL; the module-bus SPI that clocks the 74HC595s can be HAL too (it is not timing-critical the way the pulse timer is).

### 4.1 UARTs

| Logical (Rev5.4) | STM32 | TX pin | RX pin | AF | Status | Drv |
| --- | --- | --- | --- | --- | --- | --- |
| UART0 (USB-bridged console) | USART3 | PD8 | PD9 | AF7 | ⚡ VERIFY | HAL |
| UART1 | USART1 | PA9 | PA10 | AF7 — CONFLICTS with USB ID/VBUS; see note | ⚡ VERIFY | HAL |
| UART2 → AUX2 (RS232→Ethernet module) | UART4 | PC10 | PC11 | AF8 | ⚡ VERIFY | HAL |
| UART3 | UART5 | PC12 | PD2 | AF8 | ⚡ VERIFY | HAL |

| ⚡ VERIFY | USART1 on PA9/PA10 collides with the USB OTG FS ID/VBUS pins from Step 3. Resolve in CubeMX by either (a) moving USART1 to PB6/PB7 (AF7), or (b) not wiring USB ID/VBUS and freeing PA9/PA10 for USART1. The right answer depends on whether the board is bus- or self-powered — decide during the session. This plan assumes the console is USART3 (PD8/PD9) so it is unaffected either way. |
| --- | --- |

### 4.2 SPI Buses

| Bus | Role | SCK | MISO | MOSI | AF | Status | Drv |
| --- | --- | --- | --- | --- | --- | --- | --- |
| SPI1 | TFT + SD card (display module) | PA5 | PA6 | PA7 | AF5 | ⚡ VERIFY | HAL |
| SPI2 | Module bus — 74HC595 shift registers (outputs A–P) | PB13 | PB14 | PB15 | AF5 | ⚡ VERIFY | HAL |

| ℹ INFO | SPI1 (PA5/6/7) overlaps the QSPI IO2/IO3 alternate options noted in Step 6 — the QSPI plan below deliberately avoids PA6/PA7 to keep SPI1 clear. Confirm no overlap remains in CubeMX. Chip-selects (TFT_CS, SD_CS, and the 595 latch/RCK lines) are plain GPIO — see Step 7. |
| --- | --- |

### 4.3 I2C Buses

| Bus | Role | SCL | SDA | AF | Status | Drv |
| --- | --- | --- | --- | --- | --- | --- |
| I2C1 | Module bus TWI (via PCA9517D) | PB8 | PB9 | AF4 | ⚡ VERIFY | HAL |
| I2C2 | EXT1 TWI (via PCA9517D) | PB10 | PB11 | AF4 | ⚡ VERIFY | HAL |

# Step 5 — Timer Subsystem

Design Spec §5.4. The pulse generator (TIM2) and any other timing-critical timers generate LL; general clock/delay timers can be HAL. Timer channel pin choices have several valid options — verify each AF in CubeMX.

### 5.1 TIM2 — Pulse Sequence Generator (LL)

| Signal | Pin | AF / Note | Status | Drv |
| --- | --- | --- | --- | --- |
| TIM2_CH1 → LDAC pulse output | PA0 | AF1 (also PA5/PA15 possible) — the driver uses CH1 lead / CH2 trail on one pin pair | ⚡ VERIFY | LL |
| TIM2_ETR → R external trigger (module input R) | PA5 or PA15 | AF1 — pick an ETR-capable pin not used by CH1 | ⚡ VERIFY | LL |
| TIM2_CH2 → Q capture (module input Q), if used as capture | PB3 | AF1 — only if Q is wired as a TIM2 input capture; confirm intent | ⚡ VERIFY | LL |

| ⚡ VERIFY | The LDAC output and the Q/R trigger/capture must all live on TIM2 pins without colliding. TIM2 has limited pin options (mostly PA0–PA3, PA5, PA15, PB3, PB10, PB11). Map LDAC, ETR (R), and — if Q is a capture — the capture channel together in CubeMX and confirm no overlap with USB (PA11/PA12), SPI1 (PA5/6/7), or SWD (PA13/PA14). If PA5 is taken by SPI1, use PA15 for ETR. This is the tightest cluster in the whole plan — work it carefully. |
| --- | --- |

### 5.2 TIM1 — PWM Bank, 4ch @ 50 kHz, 12-bit (LL or HAL)

| Signal | Pin | AF | Status | Drv |
| --- | --- | --- | --- | --- |
| TIM1_CH1 | PE9 | AF1 | ⚡ VERIFY | LL/HAL |
| TIM1_CH2 | PE11 | AF1 | ⚡ VERIFY | LL/HAL |
| TIM1_CH3 | PE13 | AF1 | ⚡ VERIFY | LL/HAL |
| TIM1_CH4 | PE14 | AF1 | ⚡ VERIFY | LL/HAL |

| ℹ INFO | Using the PE9/11/13/14 cluster for TIM1 keeps all four PWM outputs grouped on Port E and off the crowded Port A. PA8–PA11 are the alternative TIM1 pins but PA11 is USB — so the Port E cluster is the cleaner choice here. |
| --- | --- |

### 5.3 Clock & Delay Timers

| Timer | Role | Pin(s) | Status | Drv |
| --- | --- | --- | --- | --- |
| TIM8 | Burst clock (RCR) — output | TIM8_CH1 = PC6 (AF3) | ⚡ VERIFY | LL/HAL |
| TIM3 | Continuous clock — output | TIM3_CH1 = PC6/PB4/PA6 — pick one free | ⚡ VERIFY | HAL |
| TIM4 | Continuous clock — output | TIM4_CH1 = PD12 (AF2) | ⚡ VERIFY | HAL |
| TIM5 | Low-freq / sub-Hz clock (32-bit) | TIM5_CH1 = PA0/PH10 — note PA0 may be TIM2; pick alt | ⚡ VERIFY | HAL |
| TIM7 | Delay generator (interrupt only, no pin) | none | ✓ SOLID | HAL |
| TIM6 | HAL time-base (if HAL timekeeping used) | none | ✓ SOLID | HAL |

| ⚡ VERIFY | Watch TIM8_CH1 (PC6) vs TIM3_CH1 (also can be PC6) — they cannot share a pin. Assign TIM3 to a different pin (PB4 or PA6, if free) so PC6 is TIM8's. Also TIM5_CH1's PA0 option collides with TIM2's LDAC — use PH10 for TIM5 or move TIM2 LDAC. CubeMX will flag these; the resolution is just picking the non-conflicting alternate. |
| --- | --- |

# Step 6 — QSPI Config Flash (W25Q80DV, 1 MB)

Design Spec §6.2. Single-bank (BK1) single-quad flash for MIPS config storage. Generates HAL (used with FatFs). QSPI has the most scattered AF mapping of any peripheral here — confirm every line.

| Signal | Proposed Pin | AF | Status | Drv |
| --- | --- | --- | --- | --- |
| QUADSPI_CLK | PB2 | AF9 | ⚡ VERIFY | HAL |
| QUADSPI_BK1_NCS | PB10 | AF9 — NOTE collides with I2C2 SCL (Step 4.3); move I2C2 or NCS | ⚡ VERIFY | HAL |
| QUADSPI_BK1_IO0 | PD11 | AF9 | ⚡ VERIFY | HAL |
| QUADSPI_BK1_IO1 | PD12 | AF9 — collides with TIM4_CH1 (Step 5.3); pick one | ⚡ VERIFY | HAL |
| QUADSPI_BK1_IO2 | PE2 | AF9 | ⚡ VERIFY | HAL |
| QUADSPI_BK1_IO3 | PD13 | AF9 | ⚡ VERIFY | HAL |

| ⚡ VERIFY | Two collisions are baked into the proposed values above and MUST be resolved in CubeMX: (1) QSPI BK1_NCS on PB10 vs I2C2_SCL on PB10 — move I2C2 to PF1/PF0 (AF4) or NCS to PB6/PC11; (2) QSPI BK1_IO1 on PD12 vs TIM4_CH1 on PD12 — move TIM4 to a different channel/pin. These are left visible rather than silently resolved so they get a deliberate decision during the session. The scattered AF9/AF10 QSPI mapping (confirmed against ST community references) is exactly why CubeMX must validate each line. |
| --- | --- |

# Step 7 — Plain GPIO Assignments

These are simple digital I/O with no alternate function — assign to any free pins in CubeMX after the peripherals above have claimed theirs. Grouping suggestions given for clean layout; exact pins are flexible.

### 7.1 Module Bus Discrete Lines

| Function | Count | Note |
| --- | --- | --- |
| Module inputs S–X (6 lines) | 6 | J6 inputs, direct GPIO read (Q,R are on TIM2 per Step 5.1) |
| 74HC595 latch/RCK lines | 2 | One per shift register (J4, J5 output latches) |
| 74HC595 output-enable / board-select (74HCT138 address) | 3 | PIN25/26/27 equivalent — 3-to-8 decoder address lines |
| SN74LVCC3245 direction/OE lines | up to 3 | One ~OE per module-bus buffer (J4/J5/J6) |

### 7.2 UI & Control

| Function | Count | Note |
| --- | --- | --- |
| TFT control: CS, DC, RESET | 3 | SPI1-attached display (data on SPI1) |
| SD card CS | 1 | Also on SPI1 bus, separate chip-select |
| Rotary encoder A / B / switch | 3 | Quadrature + push; optionally A/B on a TIM encoder-mode input later |
| RGB indicator LED | 3 | 3 GPIO |
| Status LEDs (ON/RX/TX/L) | 4 | 4 GPIO |
| LDAC-related / gate circuit control (IC11 successor, PWM13-equiv) | 1–2 | Depends on how the §2.2.6 gate/pulse circuit is reimplemented — may use a TIM channel or DAC instead |

### 7.3 Analog Inputs (ADC)

| Function | Count | Note |
| --- | --- | --- |
| ADC connector channels | 4 | Design Spec §2.2.7 — assign to ADC1/ADC2/ADC3 channels on dedicated analog-capable pins; choose channels on a favorable ADC instance with clean VREF+/VDDA routing |
| On-chip DAC outputs (optional, new capability) | 1–2 | H7 has 2×12-bit DACs — candidate replacement for the PWM-plus-filter analog setpoint in the §2.2.6 gate circuit; on PA4/PA5 (note PA5 contention) |

# Step 8 — FreeRTOS & Code Generation

### 8.1 FreeRTOS

- Middleware → FREERTOS → Interface = CMSIS_V2 (the standard wrapper; keeps future portability).

- Confirm SysTick is the FreeRTOS tick source (default) and that the HAL time-base is on TIM6 (Step 5.3), NOT SysTick — CubeMX prompts for this when FreeRTOS is enabled. This is the one setting that most often trips people up.

- Set the TIM2 pulse-generator NVIC priority appropriately relative to the RTOS (it is timing-critical). Note: FreeRTOS-managed interrupts must respect configMAX_SYSCALL_INTERRUPT_PRIORITY — if the pulse ISR calls no RTOS API, it can run at a higher priority than the RTOS threshold for lowest latency.

### 8.2 Per-Peripheral Driver Selector (the hybrid split)

Project Manager → Advanced Settings → Driver Selector. Set:

| Peripheral | Driver |
| --- | --- |
| TIM2 (pulse generator) | LL |
| TIM1 (PWM bank) | LL (or HAL if you prefer HAL_TIM_PWM convenience) |
| TIM8/TIM3/TIM4/TIM5 (clocks) | HAL |
| TIM6/TIM7 (timebase/delay) | HAL |
| USB / USB_DEVICE | HAL (middleware is HAL-only) |
| QUADSPI | HAL |
| SPI1 / SPI2 | HAL |
| I2C1 / I2C2 | HAL |
| USART/UART (all) | HAL |
| GPIO | LL (lightweight; fine either way) |

### 8.3 Generate

- Project Manager → set Toolchain/IDE (STM32CubeIDE, or Makefile if using your existing PlatformIO-style flow — confirm which you want).

- Generate Code. The LL-configured timers will produce LL init that your STM32PulseTimer driver's begin() layers on top of; pass the resolved TIM2 kernel clock (from Step 1) into begin().

| ℹ INFO | After generation, the pulse-generator driver (STM32PulseTimer.h/.cpp, already written) drops in on top of the LL init CubeMX produces. The generated MX_TIM2_Init() sets up the base timer; the driver's begin() applies the immediate-compare (preload-off) and ETR-trigger configuration on top. Confirm CubeMX did not re-enable compare preload. |
| --- | --- |

# Appendix — Known Conflicts to Resolve in CubeMX

Collected from the notes above; these are the deliberate collisions left visible so they get an explicit decision during the session. None are show-stoppers — each has an easy alternate.

| # | Conflict | Suggested resolution |
| --- | --- | --- |
| 1 | USART1 (PA9/PA10) vs USB OTG FS ID/VBUS (PA9/PA10) | Console is on USART3 anyway; either move USART1 to PB6/PB7 or drop USB ID/VBUS sensing |
| 2 | QSPI BK1_NCS (PB10) vs I2C2_SCL (PB10) | Move I2C2 to PF0/PF1 (AF4), or QSPI NCS to PB6/PC11 |
| 3 | QSPI BK1_IO1 (PD12) vs TIM4_CH1 (PD12) | Move TIM4 to another channel/pin (e.g. TIM4_CH2 PD13 — but that's QSPI IO3; pick a clear one) |
| 4 | TIM8_CH1 (PC6) vs TIM3_CH1 (PC6 option) | Assign TIM3 to PB4 or PA6; keep PC6 for TIM8 |
| 5 | TIM5_CH1 (PA0) vs TIM2 LDAC (PA0) | Use PH10 for TIM5, or relocate TIM2 LDAC to PA5/PA15 |
| 6 | TIM2 cluster (LDAC + ETR + Q capture) all on limited PA pins | Map together; if PA5 taken by SPI1, ETR→PA15; confirm no SWD/USB overlap |

| ✓ SOLID | None of these conflicts indicate a resource shortage — the LQFP-144 has ample pins (114 I/O). They are just cases where two attractive default pins overlap, each with a clean alternate. The purpose of surfacing them here is so the CubeMX session spends its time confirming a plan rather than discovering surprises. |
| --- | --- |

Next step after this worksheet: work through Steps 1–8 in CubeMX together, resolving the appendix conflicts as we reach them, then generate code and drop in the pulse-timer driver.