# MIPS Controller Hardware Upgrade

Design Specification: SAM3X8E → STM32H7 Migration

Baseline hardware analyzed: MIPS Rev 5.4 (ATSAM3X8EA-AU)

GAA Custom Electronics, LLC   |   Draft v0.8   |   July 2026

# Document Control

| Field | Value |
| --- | --- |
| Document | MIPS Controller Hardware Upgrade — Design Specification (SAM3X8E → STM32H7 Migration) |
| Target board revision | MIPS Rev 6.0 (new design); baseline analyzed: MIPS Rev 5.4 |
| Author / owner | Gordon Anderson, GAA Custom Electronics, LLC |
| Status | Draft — for review |
| Current version | v0.8 (July 2026) |

### Revision History

| Ver | Summary of Changes |
| --- | --- |
| v0.1 | Initial draft — reverse-engineered Rev 5.4 baseline from Eagle schematic + GAACE API reference; processor selection (STM32H743ZIT6), peripheral mapping, USB/QSPI/Ethernet subsystems, bootloader, risk register, phased plan. |
| v0.2 | Resolved J4/J5/J6 as GPIO bus (outputs A–P, inputs Q–X; Q/R as 32-bit timer capture/trigger); Ethernet clarified as external RS-232-to-Ethernet on AUX2; +5V module rail confirmed lightly loaded. |
| v0.3 | Resolved MSC read/write requirement, SD-vs-QSPI relationship, CAN not needed, erase-button intent. |
| v0.4 | Repurposed QSPI to MIPS config storage (1 MB); MSC thumb-drive backed by display-module SD card; dropped physical erase button. |
| v0.5 | Added Section 11: KiCad conversion (Stage A) + Rev 6.0 new-design roadmap (Stage B), key-parts BOM, power-architecture note, effort estimate, CubeMX guidance. |
| v0.6 | Added Section 5.1.1 pin-drive comparison for unbuffered EXT/AUX connectors. |
| v0.7 | Review pass: added silicon-revision (Rev V/Y) guidance to BOM; documented previously-missing analog/signal-conditioning circuits (IC11 gate/pulse, IC3/IC5 LDAC timing, IC1/IC2 input conditioning, U3 level shifter) as Section 2.2.6; detailed ADC input connector as Section 2.2.7; added mechanical/form-factor section (11.2.5); added this document-control page and glossary. |
| v0.8 | Added Section 5.4 (Timer Subsystem & Allocation): full TIMx allocation map, pulse sequence generator design decisions (TIM2, immediate mid-period compare, passed-compare hazard handling, ETR trigger — driver written separately), 4-channel 50 kHz 12-bit PWM bank (TIM1, with 204.8 MHz clock-tree requirement), burst/continuous clock generators (TIM8 RCR / TIM3/4/5), and FreeRTOS timer interaction. Updated risk register accordingly. |

# 1. Executive Summary

This document defines the target hardware and firmware architecture for upgrading the MIPS controller from the Atmel SAM3X8E (Cortex-M3 @ 84 MHz, as implemented on MIPS Rev 5.4) to an STM32H7-family Cortex-M7 microcontroller. It supersedes the earlier options survey (Grand Central M4 / Arduino Giga) with a part selected specifically for solderability, performance headroom, and the field-serviceability features discussed since: hand/reflow-solderable package (no BGA), a single-connector USB composite device (virtual COM port + mass-storage docs partition), and a clear path to Ethernet.

| ℹ INFO | Recommended device: STM32H743ZIT6 — Cortex-M7 @ 480 MHz, double-precision FPU, 2 MB dual-bank flash, 1 MB RAM, LQFP-144 (20×20 mm, 0.5 mm pitch), 114 GPIO. Same silicon family as the STM32H743VIT6 already in use on the Briotech UV board, stepped up to the 144-pin package once USB, QSPI, Ethernet RMII, and the full Rev 5.4 I/O set are all accounted for on one part. |
| --- | --- |

MIPS Rev 5.4 is a custom PCB with the SAM3X8E soldered directly — not an Arduino Due shield. Arduino Due pin-aliasing (PWM2–PWM13, ADn, PINnn, RXD1–3/TXD1–3, etc.) is retained in the schematic net names for firmware continuity with the existing GAACE/MIPS codebase, but there is no physical Due module to swap. The new board follows the same pattern: STM32H743 soldered directly to a new controller PCB, with the module-bus connectors, TFT/encoder UI, and JTAG header carried forward.

## 1.1 Scope of This Revision

- Processor migration: SAM3X8E (Cortex-M3, 84 MHz, LQFP-100) → STM32H743ZIT6 (Cortex-M7, 480 MHz, LQFP-144)

- Preserve: 3× module bus connectors (J4/J5/J6), EXT1/EXT2 auxiliary headers, 6-pin ADC breakout, AUX1/AUX2, 2.2" TFT + rotary encoder UI, JTAG/SWD debug header, front-panel status LEDs (ON/RX/TX/L)

- Add: USB composite device (CDC virtual COM + MSC docs/software partition) on the existing single micro-USB connector

- Add: QSPI NOR flash — backs the MSC docs partition and gives large non-volatile storage headroom for waveform/table data (mirrors the benefit called out for the Grand Central M4's onboard QSPI in the original roadmap analysis)

- Add: Ethernet PHY (RMII) — the SAM3X8E's Ethernet MAC pins exist on Rev 5.4 (PB0–PB9) but are not wired to a PHY; this revision populates one

- Add: dual-bank flash field-update scheme (A/B firmware images) via a custom bootloader entered by serial command, with the ROM USB-DFU bootloader retained as a hardware-level recovery path

- Port: GAACE_Core library set to STM32 — currently validated on SAMD21, SAMD51, SAM3X8, ESP32, and Teensy per the GAACE API Reference; STM32 is a new target

## 1.2 Key Open Items Requiring Gordon's Input

Flagged inline throughout (⚠) and collected here for convenience. Items resolved as of the 7/2026 discussion are marked ✓ and detailed in place. As of this revision, all open items from earlier drafts have been resolved.

- ✓ RESOLVED — Function of J4/J5/J6: general-purpose module I/O bus, 16 outputs (A–P, SPI-shift-register driven) + 8 inputs (Q–X, direct GPIO). Q/R are a dedicated 32-bit timer capture/external-trigger pair for the Table pulse-sequence engine, not general I/O. See Section 2.2.3.

- ✓ RESOLVED — Ethernet is implemented today via an external RS-232-to-Ethernet module on AUX2 (UART2), not native MAC/PHY. See Sections 2.2.2 and 6.3.

- ✓ RESOLVED — +5V module bus rail is lightly loaded or unused in most modules; not a regulator-sizing constraint. See Section 2.3.

- ✓ RESOLVED — the thumb-drive/MSC interface is backed by the microSD card on the display module (read-write), not QSPI flash. See Sections 5.3 and 6.1.

- ✓ RESOLVED — QSPI flash is dedicated solely to MIPS control/configuration storage, sized to 1 MB (≈ 8 Mbit part, e.g. Winbond W25Q80DV). See Section 6.2.

- ✓ RESOLVED — CAN interface is not needed. The CANTX0/CANRX0/CANTX1 pin aliases in the Rev 5.4 schematic are vestigial Due-pinout naming with no real CAN function; the corresponding pins are freely reusable as plain GPIO on the new board.

- ✓ RESOLVED — no physical ERASE button on the new board; the ROM bootloader recovery path (Section 7.1, via BOOT0) already covers hard recovery. See Section 5.3.

- Confirm whether any EXT1/EXT2/AUX1/AUX2 signals (unbuffered, driven directly by bare MCU pins) rely on the SAM3X8E's specific 15 mA/9 mA high-drive pins for something beyond ordinary logic-level signaling — e.g., directly driving an LED, opto-isolator, or small relay/MOSFET gate on the module side. See Section 5.1.1.

# 2. Current Hardware Baseline — MIPS Rev 5.4

Reverse-engineered from the supplied Eagle schematic (MIPS_Rev5_4.sch, 277 parts across 2 sheets). This section documents what exists today so the migration mapping in Section 5 has a concrete starting point.

## 2.1 Processor

| Reference | Part | Package | Notes |
| --- | --- | --- | --- |
| IC12 | ATSAM3X8EA-AU | LQFP-100 (TQFP) | Main MCU, soldered directly to PCB (not a Due module) |
| Y1 | 12 MHz crystal (KX-7, 20pF) | — | Main oscillator, drives PLL |
| Y2 | 32.768 kHz (CM8V-T1A) | — | RTC / slow clock |
| IC14 | NCP1117ST33T3G | SOT-223 | 3.3 V LDO regulator |
| IC13 | LM2734Y | SOT23-6 | Switching regulator (secondary rail) |

## 2.2 Processor I/O Map (as wired on Rev 5.4)

Full pin-by-pin net trace of IC12, grouped by function. Arduino Due-style aliases from the original schematic are preserved for cross-reference with existing GAACE/MIPS source.

### 2.2.1 Serial / Communications

| Net | MCU Pin | Function |
| --- | --- | --- |
| RX / TX | PA8 / PA9 | UART0 — primary USB-bridged serial (or debug UART) |
| RXD1 / TXD1 | PA12 / PA13 | UART1 |
| RXD2 / TXD2 | PA10 / PA11 | UART2 — routed to AUX2 connector |
| AD11(TXD3) / AD14(RXD3) | PB20 / PB21 | UART3, shared with analog input pins (mux) |
| MISO / MOSI / SPCK | PA25 / PA26 / PA27 | SPI0 — shared bus: TFT, shift registers (IC6/IC7), external module bus buffering |
| SDA0-3 / SCL0-3 | PB12 / PB13 | TWI1 — module bus I2C, gated through PCA9517D (IC15) buffer |
| SDA / SCL | PA17 / PA18 (aliased) | TWI0 — second I2C bus side of IC15 buffer, feeds EXT1 connector |
| D+ / D– / VBUS / ID | DHSDP/DHSDM/VBUS(via XUSB)/UOTGID | Native USB (device today, CDC virtual COM) |

### 2.2.2 Ethernet MAC (RMII) — unpopulated on Rev 5.4

| Net | MCU Pin |
| --- | --- |
| ETX_CLK | PB0 |
| ETX_EN | PB1 |
| ETXD0 | PB2 |
| ETXD1 | PB3 |
| ERX_DV | PB4 |
| ERXD0 | PB5 |
| ERXD1 | PB6 |
| ERX_ER | PB7 |
| EMDC | PB8 |
| EMDIO | PB9 |

| ℹ INFO | RESOLVED (Gordon, 7/2026): Ethernet on Rev 5.4 is not provided via these native MAC pins at all — it's implemented today with an external RS-232-to-Ethernet converter module connected through the AUX2 header (UART2 / TXD2-RXD2). The native MAC pins broken out on IC12 have never been populated with a PHY. See Section 6.3 for how this carries forward. |
| --- | --- |

### 2.2.3 Module Bus Connectors (J4 / J5 / J6)

Three 16-pin Harting ML16 connectors, each carrying 8 signal lines + 8 ground returns, each buffered through its own SN74LVCC3245APW octal bidirectional level-shifting transceiver (3.3 V MCU side ↔ 5 V field side). RESOLVED (Gordon, 7/2026): collectively these three connectors form MIPS's general-purpose module I/O bus — 16 digital outputs, labeled A through P in firmware, and 8 digital inputs, labeled Q through X.

| Connector | Buffer IC | Signal Source (A-side of buffer) | Function |
| --- | --- | --- | --- |
| J4 | IC8 (SN74LVCC3245APW) | IC6 — 74HC595 shift register (QA–QH) | Outputs A–H — SPI-loaded because direct GPIO pin count on the SAM3X8E wasn't sufficient for 16 dedicated output pins |
| J5 | IC9 (SN74LVCC3245APW) | IC7 — 74HC595 shift register (QA–QH) | Outputs I–P — second shift register, same SPI0 bus, independent RCK latch |
| J6 | IC10 (SN74LVCC3245APW) | Direct MCU GPIO/UART: PIN28, PIN46, PIN50, PIN51, CANTX1/IO (PB14), RXD1 (PA12) | Inputs Q–X — buffered directly to processor pins, no shift register needed since these only need to be read, not driven |

| ⚡ CRITICAL | Inputs Q and R are not general-purpose — Q is the capture input and R is the external trigger input for a single 32-bit hardware timer with capture/compare, used to generate the Table pulse-sequence engine's output. This is the same timer/capture function the original MIPS_Firmware_Analysis referred to as the SAM3X8E TC-register-direct MIPStimer/Table engine. Any STM32H743 replacement for Q/R must land on one of the H7's 32-bit general-purpose timers (TIM2 or TIM5) with input capture and ETR (external trigger) support — see Section 5.2. |
| --- | --- |

All three buffers share output-enable logic from IC4 (SN74HCT138D, 3-to-8 decoder) addressed by PIN25/PIN26/PIN27 — consistent with the README's description of “board-select addressing (up to 8 modules per system).”

### 2.2.4 Auxiliary Headers

| Connector | Pins | Signals Carried |
| --- | --- | --- |
| EXT1 | 14-pin ML14 | GND, +5V, SDA/SCL (TWI0 side, via IC15), TXD0/RXD0, TXD1, LDAC, PWM2, PWM12, PIN30, PIN47/48/49 |
| EXT2 | 14-pin ML14 | GND, +5V, MOSI/MISO/SPCK (SPI0), PWM5–PWM9, PIN25/26/27, AD14(RXD3) |
| ADC | 6-pin ML6 | GND + 4 unlabeled analog nets (N$42–N$47) — dedicated analog input breakout |
| AUX1 | 6-pin ML6 | GND, PWM13, 3 unlabeled nets shared with AUX2 |
| AUX2 | 6-pin ML6 | GND, PWM13, TXD2/RXD2 (UART2), shares 2 nets with AUX1 |

### 2.2.5 User Interface

| Item | Part | Interface |
| --- | --- | --- |
| Display | DIS1 — 2.2" TFT (ILI9340-class) | SPI0 (shared MISO/MOSI/SPCK) + dedicated CS/DC/RESET; separate SD_CS/SD_MISO/SD_MOSI/SD_SCK for the display-module microSD. All TFT/SD SPI lines are level-shifted 3.3V↔3V through U3 (74HC4050 hex buffer) |
| Rotary encoder + RGB LED | SW1 | A/B quadrature on PIN29/PIN32(SW)/PIN22, RGB LED on 3 dedicated nets |
| Status LEDs | ON / RX / TX / L (green/yellow/yellow/yellow) | GPIO-driven indicators |
| Reset / Erase | RESET, ERASE (tactile switches) | MASTER-RESET (NRSTB) and ERASE_S (PC0) — ERASE triggers a full-device flash erase, used to recover from a bad/interrupted programming session, not a field-firmware-update mechanism (see Section 5.3, 7.1) |
| Debug | JTAG (10-pin 2×5 header) | JTAG_TMS/TCK/TDI/TDO + dedicated JTAG_RESET, +3V3, GND |

### 2.2.6 Analog & Signal-Conditioning Circuits

Beyond the digital buses, Rev 5.4 carries several discrete signal-conditioning and analog circuits attached directly to MCU pins. These are easy to overlook (they're scattered across the schematic as small parts) but each is a real function Rev 6.0 must either carry forward or consciously drop — they are documented here so they aren't lost in the migration.

| Ref | Part | Function | MCU Pins Involved |
| --- | --- | --- | --- |
| IC11 + Q2/Q3 | LMV358 op-amp, BC847 NPN, FDN340P PMOS | Gate/pulse drive circuit: the LMV358 (IC11) compares a PWM13-derived setpoint against a feedback node (CMP) and drives GATE_CMD into the gate of Q3, a high-side FDN340P PMOS switch on the +5V rail (output N$73, with catch diode D6). Q2 (BC847) and an RC/resistor-network (C49, RN3 100K, R39) form the threshold/feedback path. Net effect: a controlled analog-set gated output driven from PWM13 — this is a genuine analog function, not glue logic, and its exact intended behavior should be confirmed before deciding how to replicate it on Rev 6.0. | PWM13, plus L13/CMP feedback nets |
| IC3 + IC5 | 74*1G86 (XOR), 74*1G04 (inverter) | DAC latch-timing logic: generates the ~LDAC / LDAC strobe from a combination of PWM11 and AD11(TXD3). ~LDAC also drives the RCK (latch) input of shift register IC6, tying DAC-update timing to the module-bus output latch. LDAC is exported on EXT1 pin 4. This is timing-critical — replicating the exact strobe relationship matters for synchronized DAC updates. | PWM11, AD11(TXD3/PB21) |
| IC1 / IC2 | 74*1G14 (Schmitt-trigger inverters) | Input conditioning / squaring for the AD9 (PB18) and AD10 (PB19) lines before they reach the MCU — with series resistors (R18/R19) on the outputs. Suggests these two lines carry edge/logic-type signals being cleaned up, rather than raw slow analog. | AD9 (PB18), AD10 (PB19) |
| U3 | 74HC4050 (hex buffer) | 3.3V↔3V level shifter for the TFT/SD SPI signals (SCLK, MOSI, LCD_CS, SD_CS, DC, RESET) — already noted in Section 2.2.5, listed here for completeness of the signal-conditioning inventory. | SPI0 SCK/MOSI + TFT/SD control lines |

| ⚠ OPEN Q | The IC11 gate/pulse circuit (and to a lesser extent the IC3/IC5 LDAC timing) are the two analog/timing functions most worth confirming intent on before Rev 6.0 schematic capture. On the STM32H743 side, the PWM13 setpoint can map to a timer PWM channel or a true DAC output (the H7 has two on-chip 12-bit DACs, which the SAM3X8E lacked — potentially a cleaner way to generate the analog setpoint than PWM-plus-filter), but the choice depends on what this circuit actually does in the instrument. Flag for confirmation. |
| --- | --- |

### 2.2.7 Analog Input Connector (ADC)

The 6-pin ADC header (Section 2.2.4) breaks out four analog input channels plus ground. Traced in detail:

| ADC Pin | Net | Path to MCU |
| --- | --- | --- |
| 1 | N$43 | Through series resistor R5 — to an MCU ADC-capable input |
| 2 | GND | Ground |
| 3 | N$42 | Through series resistor R3 |
| 4 | N$44 | Through series resistor R9 |
| 5 | N$45 | Through series resistor R4 |
| 6 | N$47 | Through series resistor R10 |

| ⚠ OPEN Q | These four analog inputs each go through a series resistor (R3/R4/R5/R9/R10) toward the MCU's ADC pins. The Rev 5.4 net names are auto-generated so the specific SAM3X8E ADC channel each maps to should be confirmed during Stage A capture, and — given GAA-CE's analog instrumentation focus — the STM32H743 ADC channel assignment for these deserves deliberate pin selection (the H7 has 3× 16-bit ADCs with many channels; choosing channels on a favorable ADC instance, with attention to the analog supply/reference routing, is worth doing intentionally rather than letting CubeMX auto-assign). Confirm expected input voltage range and whether any anti-alias/input filtering beyond the existing series resistors is wanted. |
| --- | --- |

## 2.3 Power Architecture (Rev 5.4)

- USB micro-B (USB1) with SP723ABG (U1) overvoltage/ESD protection on VBUS — primary power/data entry point

- NCP1117-3.3 LDO (IC14) generates the +3V3 rail for the MCU core/IO

- LM2734Y switcher (IC13) generates a secondary rail (likely feeding the +5V field rail distributed to J4/J5/J6/EXT1/EXT2/AUX1/AUX2 — confirm exact topology during schematic capture)

| ℹ INFO | RESOLVED (Gordon, 7/2026): the +5V rail delivered to the module bus connectors is lightly loaded or entirely unused in most modules today. This removes the main open question here — regulator sizing for the new board should still account for the STM32H743's higher core/IO current draw at 480 MHz versus the SAM3X8E at 84 MHz, but the field 5V rail itself is not expected to be a constraint. |
| --- | --- |

# 3. GAACE_Core Library Portability Assessment

Per the GAACE API Reference (v1.1.0), the library set is already architecture-agnostic by design and “battle-tested on SAMD21, SAMD51, SAM3X8 (Arduino Due), ESP32, and Teensy platforms.” STM32 is not in that list. This section assesses each module's likely portability effort.

| Module | Portability to STM32 | Notes |
| --- | --- | --- |
| commandProcessor | Direct — no change expected | Depends only on Arduino Stream objects; STM32duino (STM32Core) implements Stream identically. |
| ringBuffer | Direct | Pure C++ container logic, no hardware dependency. |
| charAllocate | Direct | Pure heap-arena logic, architecture-independent. |
| debug | Requires new platform branch | Explicitly has per-platform #ifdef paths for SAMD21/SAMD51/SAM3X8/Teensy (softwareReset, CPUTEMP, UUID). STM32H7 needs its own branch: NVIC_SystemReset() for reset, internal temp sensor ADC channel for CPUTEMP, 96-bit unique device ID register (0x1FF1E800) for UUID. |
| Button | Direct | Plain digitalRead()/millis(), no MCU-specific code. |
| Devices | Direct | SPI.h/Wire.h based; STM32duino implements both APIs. AD5592R/AD5593R/DAC8571/MCP4725 drivers are bus-level, not core-specific. |
| WireHelper | Direct, contingent on SerialBuffer | Wire.h based; works as-is on STM32duino. Confirm the (not-bundled) SerialBuffer dependency already compiles on ARM Cortex-M — likely yes since it already works on SAMD51/Teensy (also Cortex-M). |
| AtomicBlock | Direct — already supports Cortex-M3/M4 | Third-party library (Christopher Andrews) explicitly supports “AVR 8-bit, ARM Cortex-M3/M4, PIC32, and AVR32.” Cortex-M7 uses the same PRIMASK-based interrupt masking as M3/M4, so this should work unmodified; verify with a quick bring-up test rather than assuming. |
| Errors | Direct | Plain enum, no hardware dependency. |
| FlashFS | New platform config needed — but this is the important one | Already supports a QSPI backend (Adafruit_FlashTransport_QSPI) used for SAMD51/other QSPI-equipped boards. This is directly reusable for the STM32H7's QSPI-attached NOR flash — see Section 6.2. Board-specific pin config (flash_config.h) needs an STM32H743 entry. |

| ℹ INFO | Net assessment: roughly two-thirds of GAACE (commandProcessor, ringBuffer, charAllocate, Button, Devices, WireHelper, AtomicBlock, Errors) should port with zero or near-zero changes, since none of it touches SAM3X8E-specific registers — it was already written against Arduino Stream/SPI/Wire abstractions specifically to be portable. The two modules needing real work are debug.h (new platform branch, small and mechanical) and FlashFS's board config (new QSPI pin/clock definition for STM32H743, using an existing pattern already proven for other QSPI parts). |
| --- | --- |

This is a materially smaller lift than the MIPS application-layer migration itself (Section 5), because GAACE was built with portability as an explicit design goal while the MIPS firmware proper (per the earlier roadmap analysis) still has ~112 direct AtomicBlock call sites, SAM3X8E TC-register-direct timer code (MIPStimer), and direct OTG register access in SerialPortReset() that are not part of GAACE and do not benefit from GAACE's existing portability work.

# 4. Target Processor Selection

## 4.1 Recommended Part: STM32H743ZIT6

| Parameter | SAM3X8E (current) | STM32H743ZIT6 (proposed) |
| --- | --- | --- |
| Core | Cortex-M3 @ 84 MHz | Cortex-M7 @ 480 MHz, double-precision FPU + L1 cache (16KB I / 16KB D) |
| Performance | ≈ 1.9 DMIPS/MHz (M3) | ≈ 2.14 DMIPS/MHz × 480MHz ≈ 1027 DMIPS — roughly a 6–7× class jump, more on cache-friendly / floating-point code |
| Package | LQFP-100 (14×14mm) | LQFP-144 (20×20mm), 0.5mm pitch — hand/reflow solderable, no BGA |
| Flash | 512 KB, single bank | 2 MB, dual-bank (enables A/B field update — Section 7) |
| RAM | 96 KB | 1 MB (192 KB TCM + 864 KB user SRAM) |
| GPIO | ≈ 79 usable pins on this package | 114 usable pins — headroom for USB + QSPI + RMII + existing module bus/UI simultaneously |
| USB | Single USB OTG (device today) | 2× independent USB OTG cores — only one needed for the composite CDC+MSC device, second available for future use |
| External memory | None | Dual-mode QSPI up to 133 MHz + flexible external memory controller |
| Ethernet | MAC present, unpopulated (Section 2.2.2) | Same situation — MAC present, PHY to be added either way (Section 6.3) |

## 4.2 Why LQFP-144 over LQFP-100

The STM32H743VIT6 (LQFP-100, 82 I/O) was the part recommended earlier in this project when the scope was “replace the processor and keep existing peripherals.” Since then the scope grew to include a USB composite device, QSPI flash, and RMII Ethernet — together roughly 25 additional pins beyond the existing module bus (J4/J5/J6), EXT1/EXT2, ADC, AUX1/AUX2, TFT+encoder UI, and JTAG. Rather than fight for GPIO on the 100-pin part, the LQFP-144 STM32H743ZIT6 is the same silicon family (same core, same flash/RAM, same peripherals) in a larger, still hand/reflow-solderable package, with comfortable pin headroom for all of the above simultaneously.

## 4.3 Alternative Considered

| Part | Package | Why not selected |
| --- | --- | --- |
| STM32H743VIT6 | LQFP-100, 82 I/O | Already used on the Briotech board — attractive for tooling reuse, but pin-constrained once USB+QSPI+RMII+full module bus are all present at once. |
| STM32H723VET6 / VGT6 | LQFP-100, 550 MHz, lower cost | Cheaper and even faster clock, but same 100-pin constraint as above, and less flash/RAM than the H743. |
| Adafruit Grand Central M4 (ATSAMD51) | Module, BGA-based | Original roadmap recommendation — lowest migration risk but 4× less raw performance and no path to a hand-solderable custom board (it's a module, not a chip you place directly). |
| Arduino Giga R1 WiFi (STM32H747XI) | BGA only | Highest raw performance (dual-core, 480/240 MHz) but explicitly ruled out earlier for package (BGA) and Mbed OS/dual-core complexity. |

# 5. Peripheral & Signal Mapping Plan

Maps each function identified in Section 2 to its STM32H743 equivalent. This is a functional mapping for schematic capture, not a final pin-by-pin assignment — exact GPIO numbers should be finalized in CubeMX/pin-planner against the LQFP-144 footprint once connector placement is set.

## 5.1 Communications

| SAM3X8E Function | STM32H743 Equivalent | Migration Note |
| --- | --- | --- |
| UART0 (RX/TX, PA8/PA9) | USART1 or USART2 | Direct — Arduino Serial API compatible |
| UART1 (RXD1/TXD1) | USART3 | Direct |
| UART2 (RXD2/TXD2 → AUX2) | UART4 | Direct |
| UART3 (shared w/ analog pins) | UART5 or LPUART1 | H7 has dedicated pins available — no need to share with ADC inputs as SAM3X8E did |
| SPI0 (MISO/MOSI/SPCK — shared TFT + shift registers + EXT2) | SPI1 (dedicated to TFT) + SPI2 (dedicated to shift-register / module bus) | Recommend splitting the current shared SPI0 bus into two independent SPI peripherals now that pin budget allows — removes the TFT-vs-module-bus contention the original roadmap flagged as a general risk of shared buses under load |
| TWI1 (SDA0-3/SCL0-3, module bus, via PCA9517D) | I2C1 | Direct — PCA9517D I2C buffer/level-shifter is reusable as-is |
| TWI0 (SDA/SCL, via PCA9517D, → EXT1) | I2C2 | Direct |
| Native USB (device, CDC) | USB OTG1_FS or OTG2_FS, Device mode | Becomes composite CDC+MSC — see Section 6.1 |

### 5.1.1 Pin Drive Comparison — EXT1 / EXT2 / AUX1 / AUX2 (Unbuffered Connections)

Unlike J4/J5/J6 (Section 5.2), which are isolated from raw MCU drive characteristics by dedicated SN74LVCC3245APW level-shifters, the EXT1, EXT2, AUX1, and AUX2 headers are driven directly by bare processor pins on Rev 5.4 — SPI0's MOSI/MISO/SPCK go straight to EXT2, and most of the PWM/GPIO lines on EXT1/EXT2/AUX1/AUX2 have no buffer stage at all (the one exception is EXT1's TWI0 side, which does pass through the PCA9517D buffer, IC15). This means the SAM3X8E's and STM32H743's raw pin electrical characteristics matter directly for these four connectors, in a way they don't for the module bus.

| Parameter | SAM3X8E (current) | STM32H743 (proposed) |
| --- | --- | --- |
| Source current | 3 mA (standard pins) or 15 mA (select “high-drive” pins — fixed by silicon, not software-selectable which pins get it) | ±25 mA absolute maximum per pin; datasheet-guaranteed VOL/VOH specs are typically characterized around ±8 mA, with reduced margin above that |
| Sink current | 6 mA or 9 mA, depending on the specific pin | Same ~25 mA absolute max / ~8 mA guaranteed-spec pattern as source current |
| Total I/O budget | 130 mA DC combined across all I/O lines | Package-dependent, roughly in the 140 mA class total for similar pin-count STM32H7 variants — confirm the exact absolute-maximum-ratings figure for the ZIT6 specifically during schematic capture |
| 5V tolerance | None — SAM3X8E I/O is explicitly not 5V tolerant | Most STM32H743 GPIOs are FT (5V-tolerant) — a genuine improvement for these unbuffered headers |
| Drive strength uniformity | Non-uniform — only certain fixed pins get the 15 mA/9 mA “high-drive” option | Uniform across standard GPIOs — no equivalent “only some pins get more current” restriction |

| ⚡ CRITICAL | Per-pin drive current is comparable or slightly better on the STM32H743 for ordinary logic-level signaling, so a like-for-like digital connection (driving another chip's logic input) will be fine without any special attention. The one thing worth checking before finalizing the CubeMX pin assignment: if any of the current EXT1/EXT2/AUX1/AUX2 signals were deliberately routed to one of the SAM3X8E's specific 15 mA/9 mA high-drive pins because something on the module side draws more than a couple mA — an LED, an opto-isolator input, a small relay or MOSFET gate — that net should be identified so its replacement STM32H743 pin is confirmed adequate (which it almost certainly will be, since the H743 doesn't have SAM3X8E's per-pin restriction, but it's worth verifying against the actual downstream load rather than assuming). The STM32H743's broad 5V tolerance is a mild bonus here too — it removes a failure mode (back-driving a non-5V-tolerant input) that theoretically existed on the SAM3X8E design if any module ever drove one of these lines high with 5V logic. |
| --- | --- |

## 5.2 Module Bus (J4 / J5 / J6) — General-Purpose I/O: Outputs A–P, Inputs Q–X

Recommend preserving the existing three-connector, level-shifter-per-connector topology (SN74LVCC3245APW-class octal transceivers are still an appropriate choice for 3.3V↔5V translation on the new board). What changes is only the MCU-side source of each connector's 8 signal lines:

| Connector | Current Source (Rev 5.4) | Proposed STM32H743 Source |
| --- | --- | --- |
| J4 (outputs A–H) | 74HC595 (IC6) fed from shared SPI0 | Same 74HC595, now fed from dedicated SPI2 (freed from TFT contention per 5.1) — the STM32H743's pin budget means this shift register could alternatively be retired in favor of 8 direct GPIO, but keeping it preserves a known-good, already-proven design block |
| J5 (outputs I–P) | 74HC595 (IC7) fed from shared SPI0 | Same 74HC595, same dedicated SPI2 bus, separate latch (RCK) line as today |
| J6 (inputs Q–X) | Direct GPIO/UART (PIN28/46/50/51, PB14, PA12) | Direct GPIO — 6 of the 8 inputs (S, T, U, V, W, X) reassign 1:1 to any free H743 GPIO; Q and R require specific peripherals, below |

| ⚡ CRITICAL | Q and R must land on a 32-bit STM32H743 general-purpose timer with input capture and external trigger (ETR) support — TIM2 or TIM5 are the correct choices (both are 32-bit counters on the H7, unlike most of the other TIM instances which are 16-bit). Recommendation: Q → one of TIM2's or TIM5's input capture channels (e.g., TIM2_CH1), R → that same timer's ETR pin. This preserves the existing 32-bit capture/compare pulse-sequence generation behavior the Table engine depends on, rather than forcing a redesign of that timing-critical subsystem. Confirm during Phase 3 bring-up (Section 9) that the chosen timer instance's ETR pin is available on a package pin compatible with the new connector layout. |
| --- | --- |

Inputs S–X (the remaining 6 general-purpose inputs) have no special peripheral requirement and can be assigned to any free GPIO.

## 5.3 User Interface

| Function | Current | STM32H743 Plan |
| --- | --- | --- |
| TFT (ILI9340-class) | Shared SPI0 | Dedicated SPI1 (Section 5.1) — removes contention with module-bus shift registers |
| microSD (on display module) | Shared SPI0, via TFT connector | Same SPI1 bus as TFT (chip-select separated, as today). RESOLVED (Gordon, 7/2026): the SD interface physically lives on the plug-in display module, not the main controller board, so it isn't a controller-board design decision — it's preserved as-is for display-module compatibility. A generously-sized QSPI flash (Section 6.2) removes the need to rely on it for docs/software distribution. |
| Rotary encoder + RGB LED | 3 GPIO (quadrature) + 3 GPIO (RGB) | Direct GPIO reassignment; H7's TIM peripherals can optionally take over quadrature decode in hardware (TIM encoder mode) if lower CPU overhead is desired — optional enhancement, not required |
| Status LEDs (ON/RX/TX/L) | 4 GPIO | Direct GPIO reassignment |
| Reset / Erase buttons | NRSTB, ERASE_S (PC0) | RESOLVED (Gordon, 7/2026): no physical ERASE button needed on the new board — the ROM bootloader recovery path (Section 7.1, entered via BOOT0) already provides hard recovery from a bad/interrupted programming session, so a dedicated erase button/pin adds a component without adding capability. NRST maps directly to the H743's dedicated reset pin; BOOT0 should remain accessible (e.g., a solder jumper or small header, not necessarily a customer-facing button) for that recovery path. |
| JTAG/SWD | Full JTAG (TMS/TCK/TDI/TDO + reset) | SWD (SWDIO/SWCLK) is sufficient and standard for STM32 — 2-wire instead of 4, frees pins; full JTAG can still be wired if multi-vendor debug tooling requires it |

## 5.4 Timer Subsystem & Allocation

MIPS makes heavy use of hardware timers — the current SAM3X8E design uses 8 in total. The STM32H743 has far more timer resource than the SAM3X8E, so every existing use is covered with substantial headroom. Because STM32 timers are not interchangeable (different bit widths, channel counts, and special features like external-trigger and repetition-counter support), each MIPS timer function is deliberately mapped to an appropriate TIM instance rather than assigned arbitrarily. This section defines that allocation so the functions do not collide as they are ported.

### 5.4.1 Timer Allocation Map

| Timer | Width / Type | MIPS Function | Key feature used |
| --- | --- | --- | --- |
| TIM2 | 32-bit GP | Pulse sequence generator (the one PSG) | 32-bit range (µs pulse within 100s-of-ms period) + ETR hardware trigger. See 5.4.2. |
| TIM1 | 16-bit advanced (4ch) | PWM bank — 4 channels, shared 50 kHz base, 12-bit | 4 independent-duty compare channels on one time-base. See 5.4.3. |
| TIM8 | 16-bit advanced | Burst clock generator (defined N-cycle bursts) | Repetition counter (RCR) — hardware emits exactly N cycles then stops, zero CPU. See 5.4.4. |
| TIM5 | 32-bit GP | Low-frequency clock generation (incl. sub-Hz if ever needed continuously) | 32-bit range for very low frequencies. Available; not permanently locked unless a continuous sub-Hz clock is required. |
| TIM3, TIM4 | 16-bit GP (4ch each) | High-frequency continuous clock generators | Output-compare / PWM output mode |
| TIM7 | 16-bit basic | Delay generator (interrupt-only, no pin) | Counter + update interrupt only — exactly what a basic timer is for |
| TIM6 | 16-bit basic | HAL time-base (if HAL timekeeping is used) | Keeps HAL off SysTick so FreeRTOS can own it (5.4.5) |
| SysTick | core timer | FreeRTOS scheduler tick | Dedicated ARM core timer — costs zero TIM peripherals |
| TIM12–17, LPTIM1–5 | assorted | Spare / future growth | Substantial unused headroom remains |

| ℹ INFO | Delay generators are interrupt-only (no pin output) and used one at a time, 2 max — TIM7 covers the primary need, with a second basic/GP timer available as backup if two are ever needed concurrently. Because the SAM3X8E used 8 timers total and the H743 allocation above uses well under half its timer resource for the same functions, the migration has ample room for the remaining legacy uses plus future features. |
| --- | --- |

### 5.4.2 Pulse Sequence Generator (TIM2) — Design Decisions

The pulse sequence generator drives the MIPS DAC-update cycle: firmware loads AD5668 DAC values (and digital output words), the timer fires a narrow LDAC pulse to update all outputs simultaneously on the LDAC falling edge, then firmware prepares the next step. It is a direct port of the SAMD51 TCC-based timer, adapted to TIM2 with two behavioral changes that are critical to get right:

| ⚡ CRITICAL | IMMEDIATE mid-period compare updates (not buffered). The SAMD51 version buffered compare/period updates to take effect at the next overflow. MIPS requires the opposite: the LDAC pulse position must be settable on the fly, mid-period, and take effect immediately within the currently-running period — because a single period can be hundreds of milliseconds long and the firmware moves the pulse position while that period is still counting. Therefore output-compare preload is DISABLED (OCxPE = 0) so compare-register writes go live immediately. This is the single most important design decision in the pulse generator and must not be reverted to a buffered scheme. |
| --- | --- |

Passed-compare hazard handling: with preload off and a long period, if firmware moves the compare to a value the counter has already passed, a normal match will not fire again until the next period — a missed pulse. The driver detects this (writes the compare, then re-reads the live counter; write-then-read order makes an observed count ≥ target authoritative) and applies a selectable policy — fire-now (default: force one LDAC pulse immediately so the step is not lost), skip, or wrap-to-next-period — reporting back which path was taken.

- Timer: TIM2 (32-bit; TIM5 is the identical drop-in alternative if TIM2 is needed elsewhere)

- LDAC output: narrow active-low pulse (lead-edge/trail-edge compare pair defines width precisely regardless of period length), default ~1 µs, well above the AD5668's ~20 ns minimum LDAC low width

- External trigger: hardware ETR slave-mode reset — an external edge on the Q/R trigger input (Sections 2.2.3 / 5.2) resets and restarts the counter in silicon with zero interrupt latency, replacing the SAMD51 version's software-interrupt trigger

- Prescaler: full 16-bit integer divider (1–65536), far finer than the SAMD51's 8 fixed power-of-two steps, giving user-programmable clock rates per application

- Callbacks: start / mid (LDAC instant) / terminal (period rollover — where the sequence engine loads the next step) — all three retained from the SAMD51 API

- Framework: STM32 LL (Low-Layer) driver, consistent with a native STM32Cube project (not the Arduino framework)

### 5.4.3 PWM Bank (TIM1) — 4 Channels, 50 kHz, 12-bit

Four PWM channels sharing a common 50 kHz base frequency with independent duty cycles come from a single advanced-control timer's four compare channels (TIM1). For true 12-bit resolution (4096 counts) at 50 kHz, the timer time-base must count at 4096 × 50 kHz = 204.8 MHz.

| ⚡ CRITICAL | Clock-tree requirement: achieving the full 12-bit resolution at 50 kHz requires the TIM1 kernel clock to reach ≈ 204.8 MHz. At a 200 MHz timer clock the resolution is 4000 counts (≈ 11.97 bits — essentially but not exactly 12-bit); the H7's higher timer domains can reach 204.8–240 MHz, giving the full 4096 counts with margin. This should be set deliberately in the CubeMX clock-tree configuration and validated, not left to chance — flag it as a specific clock requirement during pin/clock planning (Section 11.2.1). |
| --- | --- |

### 5.4.4 Clock Generators (TIM8 burst, TIM3/TIM4/TIM5 continuous)

MIPS uses two categories of clock generation, which map to different hardware capabilities:

- Burst clocks (low-frequency, a defined number of cycles then stop): mapped to TIM8, using its repetition counter (RCR) so the hardware emits exactly N cycles and stops with zero CPU involvement — the same 'exactly N cycles then stop' pattern as the pulse generator's multishot mode, but done entirely in hardware. Assumed to be transient (set up → fire N cycles → done) rather than continuously running.

- Continuous clocks: higher-frequency continuous outputs on TIM3/TIM4 (16-bit, output-compare/PWM mode). If a continuous very-low-frequency (sub-Hz) clock is ever needed in the background, the 32-bit TIM5 provides the range for it.

### 5.4.5 RTOS Timer Interaction

This revision introduces an RTOS (FreeRTOS, consistent with the GAACE portability and risk assessments in Sections 3 and 8). Its impact on timer allocation is minimal and worth stating explicitly so no general-purpose timer is accidentally consumed:

- FreeRTOS uses the ARM Cortex-M core SysTick timer for its scheduler tick — this is a dedicated core timer, not one of the TIMx peripherals, so the RTOS costs zero general-purpose timers

- The one caveat: if HAL timekeeping (HAL_Delay, etc.) is used, HAL also wants a time-base; the standard resolution is to give HAL its own basic timer (TIM6) so FreeRTOS keeps SysTick uncontested. If HAL timekeeping is not used, TIM6 is freed as well

- The pulse-generator ISR (TIM2) is timing-critical; its NVIC priority should be set appropriately relative to the RTOS and the rest of MIPS during system init

# 6. New Subsystems

## 6.1 USB Composite Device (CDC + MSC)

Single micro-USB (or USB-C, if changing connector — no functional reason not to, worth a look) connector, one STM32H743 USB OTG core (OTG1_FS or OTG2_FS — internal PHY, no external parts needed for full-speed), configured in Device mode presenting two interfaces to the host:

- CDC interface — the virtual COM port MIPS uses today for serial control, unchanged from the host PC's perspective

- MSC interface — presents as a removable drive backed by the microSD card on the display module (Section 5.3), read-write, pre-loaded at manufacturing time with docs/software

| ℹ INFO | STM32CubeH7 ships a ready-made USB_Device CDC_MSC composite class example — this is a supported, documented ST configuration, not a from-scratch build. |
| --- | --- |

| ℹ INFO | RESOLVED (Gordon, 7/2026): the thumb-drive/MSC interface is backed by the microSD card already present on the display module, not by QSPI flash. Practical implications: (1) the MSC class driver's block-device layer talks FatFs-over-SPI to the SD card via the same SPI1 bus as the TFT (Section 5.1/5.3), using a standard SD-over-SPI FatFs diskio driver rather than GAACE's QSPI-oriented FlashFS transport; (2) MSC storage capacity is now whatever SD card is fitted (typically far larger than any QSPI part would be, at lower cost); (3) since the SD card is on the display module rather than the main controller board, MSC docs content is tied to that module being present — consistent with how the system already ships today (docs/software have always traveled with the SD-card-equipped display hardware). |
| --- | --- |

## 6.2 QSPI NOR Flash — MIPS Control Configuration

RESOLVED (Gordon, 7/2026): QSPI flash is no longer tasked with docs/thumb-drive storage (that's now the SD card, Section 6.1). Its sole job is small, fast, always-on non-volatile storage for MIPS control/configuration information — the equivalent of today's default.cfg and any other persistent settings, plus headroom for calibration data and future config growth. 1 MB is confirmed as plenty for this.

| Item | Recommendation |
| --- | --- |
| Flash part | A small QSPI NOR sized around 8 Mbit / 1 MB — e.g., Winbond W25Q80DV (SOIC-8, 1 MB, well-supported, inexpensive). No need to size up further given the confirmed 1 MB target; this keeps cost and board area minimal. |
| Filesystem | FatFs, via GAACE's existing FlashFS module (Section 3), which already has a QSPI transport backend (Adafruit_FlashTransport_QSPI) — needs a new board-config entry for STM32H743, not a new transport. This is a much smaller/simpler integration than the earlier docs-partition plan, since it's a single-purpose config store rather than a customer-facing filesystem. |
| Partitioning | Single partition — app-writable, used directly by MIPS firmware for config/calibration storage. Not exposed to the USB host at all (that's the SD card, Section 6.1). |

## 6.3 Ethernet

RESOLVED (Gordon, 7/2026): Ethernet on MIPS today is not native — it's provided by an external RS-232-to-Ethernet converter module connected through the AUX2 header (UART2 / TXD2-RXD2). The SAM3X8E's native Ethernet MAC pins (Section 2.2.2) have never actually been used. This changes the recommendation from “add a PHY” to a simpler choice between two paths:

| Option | Description | Recommendation |
| --- | --- | --- |
| A — Carry forward as-is | Reserve a UART on the new board (e.g., USART3 or UART4) routed to AUX2, exactly as today, and continue using the same external RS-232-to-Ethernet module. | Recommended default — zero new hardware risk, zero new firmware (no lwIP stack, no PHY bring-up), and it's a proven, already-working part of the system. Costs nothing to carry forward and doesn't block anything else in this revision. |
| B — Add native Ethernet (RMII + PHY) | Populate an RMII PHY (e.g., LAN8720A) on the STM32H743's native MAC, add magnetics + RJ45, integrate lwIP. | Optional future enhancement, not required by anything else in this design. Only worth doing if there's a concrete reason to want higher throughput or lower latency than the RS-232 bridge provides, or to free up the AUX2 UART for another purpose. |

| ℹ INFO | Given Option A carries no cost or risk and Option B is pure net-new scope with no identified driver, this document proceeds on the assumption that Ethernet continues via the external RS-232-to-Ethernet module unless there's a specific reason to change that — flag if that's not the case. |
| --- | --- |

# 7. Field Firmware Update Architecture

Requirement: field firmware updates over the same USB virtual COM port already used for MIPS control, without requiring the customer to use separate DFU tooling or drivers under normal circumstances.

## 7.1 Two-Tier Bootloader Strategy

| Tier | Mechanism | When Used |
| --- | --- | --- |
| Primary — custom CDC bootloader | A small bootloader region listens on the same USB CDC virtual COM port already used for MIPS serial control. Entered via a serial command (e.g. LOADFW) or a magic value written to a backup-domain register before reset. Receives a new firmware image over that same port, writes it to flash, verifies, and jumps to it. | Normal field updates — no new cables, drivers, or tools for the customer |
| Recovery — ROM USB-DFU bootloader | STM32H743's factory-programmed System Memory bootloader, entered via the BOOT0 pin or STM32CubeProgrammer/dfu-util. Not written by GAA — always present in silicon. | Fallback only, if the primary bootloader itself is ever damaged or a from-scratch recovery is needed. Keep BOOT0 accessible on the new board (even if customers never touch it) specifically for this reason. |

## 7.2 Dual-Bank A/B Update Scheme

The STM32H743's 2 MB flash is organized as two independently-writable 1 MB banks with read-while-write support. This enables a genuine A/B update scheme that the single-bank SAM3X8E (and the SAMD51/Grand Central alternative evaluated earlier) cannot offer:

- New firmware image is written into the inactive bank while the active bank continues running unaffected

- CRC/checksum verification of the received image before it is marked bootable

- If a field update is interrupted (USB dropout, power loss, bad transfer), the untouched bank still boots — the instrument cannot be bricked by an interrupted update

- A boot-bank selector flag (e.g., in backup SRAM or a dedicated flash sector) determines which bank the bootloader jumps to on the next reset

| ⚡ CRITICAL | This dual-bank scheme is a meaningfully safer field-update story than either SAM3X8E or SAMD51 can offer (neither has dual-bank flash) and directly addresses the practical risk of an interrupted update in a fielded instrument at a customer site with no local support. |
| --- | --- |

## 7.3 Command Flow (Consistent with the Existing GAACE commandProcessor Protocol)

Recommend implementing update-trigger and status commands as ordinary GAACE commandProcessor entries (Section 3), so they appear in GCMDS/HELP like any other MIPS command and require no new tooling on the host side beyond a terminal or simple script:

| Command | Purpose |
| --- | --- |
| LOADFW | Requests entry into bootloader mode on next reset (sets a flag in backup SRAM), then triggers a software reset |
| GFWBANK | Reports which flash bank is currently active and its firmware version/CRC |
| GFWSTATUS | Reports last update result (success / CRC fail / interrupted) for field diagnostics |

# 8. Migration Risk Register

Adapted from the original roadmap analysis's risk framework, applied specifically to the SAM3X8E → STM32H743 path with the Rev 5.4 signal map from Section 2.

| Area | Risk | Detail |
| --- | --- | --- |
| Timer / PWM | MEDIUM | STM32H7 uses TIM (Advanced/General purpose) timers, not SAM TC channels — timer-driven code requires a rewrite. The full timer allocation is now specified in Section 5.4. The one item needing deliberate attention is the clock tree: true 12-bit PWM at 50 kHz needs the TIM1 kernel clock at ≈ 204.8 MHz (Section 5.4.3), which must be set in CubeMX rather than assumed. |
| Pulse sequence generator (TIM2) | MEDIUM | Ported from the SAMD51 timer to TIM2 with two critical changes: compare preload DISABLED for immediate mid-period updates, and explicit passed-compare hazard handling (Section 5.4.2). Driver is written; the immediate-compare + hazard path is the key thing to scope-verify on hardware with a real long-period/short-pulse sequence. |
| SPI splitting (TFT vs. module bus) | LOW-MEDIUM | Recommended change (Section 5.1/5.3) — splitting the currently-shared SPI0 bus into two independent peripherals is new work, but removes a contention risk rather than introducing one. |
| AtomicBlock (112 sites in MIPS proper) | LOW | AtomicBlock library already supports Cortex-M3/M4 PRIMASK semantics; Cortex-M7 uses the same interrupt architecture. Verify with bring-up testing rather than assuming, but no rewrite expected. |
| Flash / NVM (config storage) | MEDIUM | Replace any direct SAM3X8E flash-write API usage with the H7's flash sector API, or better, the QSPI-backed FatFs volume (Section 6.2) for a cleaner, more portable path. |
| USB serial reset (SerialPortReset()) | MEDIUM-HIGH | OTG register sequence is SAM3X8E-specific per the original firmware analysis; needs a rewrite against the H7's USB OTG peripheral. Becomes somewhat more involved with the composite CDC+MSC device (Section 6.1) than a simple CDC-only port would be. |
| GAACE library port | LOW | Per Section 3, the majority of GAACE is already portable by design; only debug.h and FlashFS's board config need new STM32H743-specific branches. |
| Ethernet | NONE (out of scope by default) | Carried forward unchanged via the existing external RS-232-to-Ethernet module on AUX2, per Section 6.3 — no native MAC/PHY work needed unless that default is changed. |
| Library ecosystem (TFT, external ADC/DAC drivers) | LOW | STM32duino has broad community support; Devices.h drivers (AD5592R/AD5593R/DAC8571/MCP4725) are bus-level and platform-independent already. |

# 9. Phased Implementation Plan

Sequenced to validate risk in ascending order, consistent with the phased approach in the original roadmap analysis, adapted to this board's actual scope.

| Phase | Work |
| --- | --- |
| 0 — Schematic capture kickoff | All open items resolved (Section 1.2) — proceed directly to schematic capture using this document's mappings |
| 1 — Bring-up board | STM32H743ZIT6 minimum viable board: power, clock, SWD/JTAG debug header, one UART. Validate GAACE core modules (commandProcessor, ringBuffer, AtomicBlock) on bare hardware. |
| 2 — Peripheral layer | Port SPI (split TFT/module-bus buses), I2C (module bus + EXT1), UART set, PWM/TIM including the TIM2/TIM5 32-bit capture/ETR pair for Q/R. Bring up TFT + rotary encoder UI. |
| 3 — Module bus validation | Bring up J4/J5/J6 (outputs A–P, inputs Q–X) with real module boards (start with DCbias, the most-used module per the original roadmap's suggested sequence), with particular attention to validating the Q/R timer capture/trigger behavior against the existing Table pulse-sequence engine. |
| 4 — USB composite + QSPI | Bring up USB CDC+MSC composite device and QSPI-backed FatFs volume; validate docs partition contents survive re-enumeration and power cycling. |
| 5 — Bootloader | Implement dual-bank A/B update scheme and LOADFW/GFWBANK/GFWSTATUS commands (Section 7); validate interrupted-update recovery. |
| 6 — Ethernet bridge carry-over | Wire the chosen UART to AUX2 for the existing external RS-232-to-Ethernet module; confirm unchanged operation. Native Ethernet only if Option B (Section 6.3) is later chosen. |
| 7 — Full module integration | Port remaining module drivers (RFdriver, Twave, ESI, Filament, ARB, FAIMS, FAIMSfb, Table engine) per the sequence already validated in the original roadmap analysis — that ordering logic (simple modules first, ARB/FAIMS/Table last due to timing-critical ISRs) still applies. |
| 8 — System integration & validation | Full system test against representative module set; field-update rehearsal; sign-off. |

# 10. Summary of Open Questions

Nearly all open questions raised in earlier drafts of this document have been resolved as of the 7/2026 discussions with Gordon; see Section 1.2 for the consolidated list of resolutions, each with a pointer to the section documenting it in full (Sections 2.2.2, 2.2.3, 2.3, 5.2, 5.3, 6.1, 6.2, 6.3). One item remains open:

- Confirm whether any EXT1/EXT2/AUX1/AUX2 signals rely on the SAM3X8E's specific high-drive pins for anything beyond ordinary logic-level signaling, so the corresponding STM32H743 pin assignment can be double-checked against the actual downstream load (Section 5.1.1).

| ℹ INFO | This document is effectively ready to move from draft to a schematic-capture-ready specification — the one remaining item above is a quick confirmation, not expected to change the overall architecture. |
| --- | --- |

# 11. KiCad Conversion & Rev 6.0 Development Roadmap

Two distinct efforts, sequenced back to back: (A) port the existing Rev 5.4 design from Eagle to KiCad as a faithful, functionally-unchanged conversion, then (B) develop Rev 6.0 as a new KiCad design built on that converted base, implementing everything specified in Sections 2–7 of this document. Keeping these separate — converting first, then redesigning — gives a known-good, DRC-clean KiCad baseline to diff against, rather than doing an Eagle-to-KiCad conversion and a full processor swap in the same pass.

## 11.1 Stage A — Eagle-to-KiCad Conversion of Rev 5.4

Goal: a KiCad 8.x project (schematic + PCB) that is electrically and mechanically identical to the existing Rev 5.4 board — no functional changes, just a tooling migration. This becomes the reference design Rev 6.0 is built from.

| Step | Detail |
| --- | --- |
| 1. Native Eagle import | KiCad's built-in importer (File → Import → Non-KiCad Project, both schematic .sch and any board .brd) handles the bulk of the geometry, net, and part-instance translation automatically. This is a good first pass, not a finished result. |
| 2. Library reconciliation | The Rev 5.4 schematic pulls from several Eagle libraries: rcl, supply1/2, 74xx-eu, 74xx-little-us, con-harting-ml, jumper, resistor-dil, SparkFun-Electromechanical, SparkFun-Connectors, Texas Instruments_By_element14_Batch_1, zetex, transistor-npn, linear, varistor, wuerth-elektronik, diode, bibaja, BaySpec, microbuilder, frames, and two custom libraries: SmartPrj and GAAextensions (the latter includes the 2.2" TFT part, the erase-recovery part, and a custom NPN transistor symbol). KiCad's importer will bring across geometry but not always clean, reusable KiCad symbols/footprints — each part needs a pass to confirm it landed on a correct KiCad symbol (or standard KiCad library equivalent) rather than an orphaned imported blob. This is almost certainly the single largest time sink in Stage A, and is worth doing carefully since Rev 6.0 reuses most of these same parts (module bus buffers, shift registers, decoder, TFT, encoder). |
| 3. Custom library audit (SmartPrj / GAAextensions) | Since these are your own libraries, check whether KiCad equivalents already exist from the ongoing Eagle→KiCad migration mentioned in other projects (e.g., BOM Tools work) — if the TFT, erase-recovery, and custom-NPN symbols/footprints have already been ported for another board, reuse them directly rather than re-drawing. |
| 4. Netlist/ERC verification | Run KiCad's Electrical Rules Check and manually spot-check a sample of nets (start with the ones this document already traced in Section 2 — IC12/SAM3X8E pinout, J4/J5/J6, EXT1/EXT2) against the original Eagle schematic to confirm the import didn't silently drop or merge any nets. |
| 5. PCB footprint/layer verification | If also converting the board file (not just schematic): verify footprint-to-symbol association survived import, confirm layer stack and design rules match the original fab spec, run DRC clean. |

| ℹ INFO | Since KiCad migration (from Eagle) is already underway for other GAA-CE projects, some of the custom-library groundwork (SmartPrj / GAAextensions equivalents) may already exist — worth checking existing KiCad libraries before re-deriving these from scratch. |
| --- | --- |

## 11.2 Stage B — Rev 6.0 New Design

Built from the Stage A KiCad baseline. This is where the STM32H743ZIT6 migration and all of Sections 5–7's specifications get implemented as an actual schematic and board.

### 11.2.1 Recommended Bill of Materials — Key Parts

| Function | Part Number | Package | Notes |
| --- | --- | --- | --- |
| MCU | STM32H743ZIT6 | LQFP-144, 20×20mm, 0.5mm pitch | Section 4 recommendation — Cortex-M7 @ 480MHz, 2MB dual-bank flash, 1MB RAM. Specify current silicon (Rev V): Rev Y silicon is capped at 400MHz (the 480MHz VOS0 voltage-scaling mode isn't available on Rev Y). Rev V has been the only production silicon since 2019, so current DigiKey stock will be Rev V, but the orderable part number does not encode die revision — confirm on first-article boards by reading REV_ID in the DBGMCU_IDC register (0x2003 = Rev V, 0x1003 = Rev Y) over SWD or at firmware boot. |
| Config flash (QSPI) | Winbond W25Q80DV | SOIC-8 | 1MB, MIPS control/config storage only (Section 6.2) |
| Module bus level shifters (×3) | SN74LVCC3245APW | TSSOP-20 | Reused as-is from Rev 5.4 (IC8/IC9/IC10 equivalents) for J4/J5/J6 |
| Output shift registers (×2) | SN74HC595 or SN74LVC595A | SOIC-16 / TSSOP-16 | Reused function from Rev 5.4 (IC6/IC7) for outputs A–H and I–P; LVC595A is a native-3.3V drop-in upgrade worth considering over the HC variant |
| Board-select decoder | SN74HCT138D | SOIC-16 | Reused as-is from Rev 5.4 (IC4) |
| I2C bus buffer | PCA9517D | SOIC-8 | Reused as-is from Rev 5.4 (IC15) for the module-bus TWI buffering |
| HSE crystal | 25 MHz, ±10ppm or better | 3.2×2.5mm SMD | Recommend 25MHz over the Rev 5.4's 12MHz — a standard, well-supported choice for STM32H7 clock trees that cleanly derives both the 480MHz core PLL and exact 48MHz USB clock without oddball PLL fractional dividers |
| LSE crystal (RTC) | 32.768 kHz | 3.2×1.5mm SMD | Same function as Rev 5.4's Y2; reuse the same or equivalent part |
| USB connector | USB-C receptacle (or micro-USB for parts commonality with Rev 5.4) | SMD | USB-C recommended for mechanical robustness and future-proofing; functionally either works since only USB 2.0 FS signaling is needed |
| USB ESD/overvoltage protection | USBLC6-2SC6 or equivalent to Rev 5.4's SP723ABG | SOT-666 / SC-70 | Standard USB D+/D–/VBUS protection |
| 3.3V regulation | TBD — see Section 11.2.2 power note | — | Open engineering decision, not yet finalized (see below) |
| Debug connector | Standard 10-pin 0.05" SWD/JTAG (ARM Cortex debug) or Tag-Connect TC2050 | 2×5 header or edge pads | SWD-only needs just 4 signals (SWDIO/SWCLK/GND/3V3) plus optional reset/SWO; Tag-Connect saves board space if preferred |

### 11.2.2 Power Architecture — Needs Engineering Attention

| ⚡ CRITICAL | This is the one area of Rev 6.0 not yet fully specified in this document and worth flagging before schematic capture: the STM32H743 at 480MHz draws meaningfully more current than the SAM3X8E at 84MHz (typically several hundred mA depending on active peripherals, versus roughly 100mA class for the SAM3X8E), and the H7 also requires its core supply (VCORE) to come from either an internal LDO (simple, just needs VCAP1/VCAP2 decoupling capacitors, less efficient) or an external/internal SMPS step-down (more efficient, needs an external inductor, more design work). Recommend starting with the internal LDO configuration for simplicity — it's the lower-risk path for a first Rev 6.0 spin — and evaluating the SMPS option later only if thermal or efficiency data from bring-up shows a real need. Separately, the existing NCP1117-3.3 LDO topology from Rev 5.4 should be re-evaluated for the new current draw; an LDO burning off USB's 5V to 3.3V at several hundred mA dissipates real heat, so a synchronous buck (e.g., a small 3.3V buck regulator in the same class as the Rev 5.4's existing LM2734Y switcher) may be the better choice for the main 3.3V rail even though it adds a small inductor to the BOM. |
| --- | --- |

### 11.2.3 Schematic Sections to Draft

- MCU core: STM32H743ZIT6 with decoupling per ST's standard application circuit, VCAP1/VCAP2 for internal VCORE LDO, BOOT0 accessible via jumper/header (Section 5.3)

- Clock: 25MHz HSE + load caps, 32.768kHz LSE + load caps

- Power: 3.3V main rail regulator (Section 11.2.2), USB VBUS protection

- USB composite: OTG FS internal-PHY connection (D+/D–/VBUS/ID), USB-C or micro-USB connector, ESD protection

- QSPI config flash: W25Q80DV wired to the H743's QSPI peripheral (Section 6.2)

- Module bus (J4/J5/J6): SPI2 → dual 74HC595 → dual SN74LVCC3245APW → J4/J5 (outputs A–P); direct GPIO → SN74LVCC3245APW → J6 (inputs Q–X, with Q/R on TIM2 or TIM5 capture/ETR per Section 5.2); SN74HCT138D board-select decoder shared across all three

- EXT1/EXT2/ADC/AUX1/AUX2: direct carry-forward per Section 5.1 mapping

- UI: TFT + microSD on dedicated SPI1 (Section 5.3), rotary encoder + RGB LED, status LEDs

- AUX2 UART carried forward to the external RS-232-to-Ethernet module unchanged (Section 6.3)

- Debug: SWD header (Section 5.3)

### 11.2.4 PCB Layout Considerations

- LQFP-144 at 0.5mm pitch is a step up in routing density from Rev 5.4's LQFP-100 SAM3X8E — plan for a 4-layer stackup at minimum (Rev 5.4 layer count should be confirmed during Stage A conversion) to get clean power/ground planes under the MCU

- USB D+/D– as a controlled-impedance differential pair (90Ω differential), kept short and away from noisy digital switching nets

- QSPI traces (4 data + clock) kept short and length-matched where practical, though at these clock rates and board sizes this is a minor concern relative to USB

- Keep the existing ADC connector / op-amp (IC11-equivalent) analog section physically separated from the new higher-speed digital sections (USB, QSPI) more so than was necessary on the 84MHz SAM3X8E design

- Decoupling: STM32H7 application notes specify a fairly involved decoupling network (multiple values across VDD/VDDA/VREF+/VCAP pins) — follow ST's reference layout guidance directly rather than reusing the SAM3X8E's simpler decoupling scheme

### 11.2.5 Mechanical / Form-Factor Constraints

Rev 6.0 is not a free-form board — it has to physically drop into the existing MIPS mechanical context. These constraints should be captured explicitly during Stage A (from the Rev 5.4 board file) and treated as fixed inputs to Rev 6.0 layout:

- Board outline and mounting-hole positions: must match Rev 5.4 (or the enclosure it installs into) unless the enclosure is also being revised. Confirm exact dimensions and hole pattern from the Rev 5.4 .brd during Stage A conversion.

- Display-module interface: the plug-in display module (which carries the microSD used for the MSC/thumb-drive interface, Sections 5.3/6.1) mates to the controller through a specific connector at a specific position — this mating connector's footprint and placement must be preserved so existing display modules remain compatible.

- Module bus connector positions (J4/J5/J6, Harting ML16): if the controller mounts into a backplane or fixed harness, these connector positions may be mechanically constrained, not just electrically — confirm whether they can move or must stay put.

- EXT1/EXT2/ADC/AUX1/AUX2 header positions: confirm whether any of these mate to fixed internal wiring/harnesses that constrain their placement.

- USB connector position: if the enclosure has a fixed cutout for the existing USB connector, the Rev 6.0 connector (whether kept as micro-USB or changed to USB-C, Section 11.2.1) must align to it — a connector-type change may require an enclosure cutout change, worth confirming before committing to USB-C.

- Front-panel elements: TFT window, rotary encoder shaft, and status-LED positions are enclosure-constrained if the existing enclosure is retained.

| ⚠ OPEN Q | Whether the Rev 6.0 board must be a drop-in mechanical replacement for Rev 5.4 (same outline, same connector positions, same enclosure) or whether the enclosure/mechanicals can also be revised is itself a decision worth making explicitly — it significantly affects layout freedom. Default assumption for this document: preserve Rev 5.4 mechanicals unless stated otherwise. |
| --- | --- |

## 11.3 Effort Estimate

Estimates assume one experienced engineer (the profile implied by this project's existing work — comfortable with KiCad, STM32CubeMX, and mixed analog/digital PCB design) working through each stage. Ranges reflect the uncertainty inherent in library conversion work and first-spin power architecture decisions.

| Stage | Task | Estimated Effort |
| --- | --- | --- |
| A | Eagle import + library reconciliation (Section 11.1, steps 1–3) | 3–6 days — wide range mainly driven by how much of the SmartPrj/GAAextensions KiCad library work already exists from other in-progress migrations |
| A | ERC/DRC verification, net spot-checks (steps 4–5) | 1–2 days |
| A subtotal |  | ≈ 1–1.5 weeks |
| B | CubeMX pin/clock configuration (see your question below) | 0.5–1 day — fast once the functional mapping in Sections 5–7 of this document is already decided, which it is |
| B | Schematic capture (Section 11.2.3 items) | 1–2 weeks — most blocks are direct carry-forward from Rev 5.4 (already-proven module bus, TFT, encoder circuits); the genuinely new work is the MCU core circuit, USB composite, QSPI, and the power architecture decision (11.2.2) |
| B | PCB layout (Section 11.2.4) | 1–2 weeks — the LQFP-144 fine-pitch routing and decoupling network are the main drivers of time here versus Rev 5.4's simpler LQFP-100 layout |
| B | DRC/DFM cleanup, BOM finalization, fab prep | 2–4 days |
| B subtotal |  | ≈ 3–5 weeks |
| Total, Stages A+B | Design work only — excludes firmware bring-up, which is covered separately in Section 9's phased plan | ≈ 4–6.5 weeks |

| ℹ INFO | This estimate covers hardware design only (schematic + PCB, through fab-ready files) — it does not include the firmware bring-up and module validation work in Section 9, which is a separate, larger effort typically run partially in parallel once boards are back from fab. The single biggest source of schedule variance is Stage A's library reconciliation (Section 11.1) — if the custom KiCad libraries already exist from other ongoing migration work, this estimate could come in toward the low end across the board. |
| --- | --- |

| ⚡ CRITICAL | Yes, run STM32CubeMX (or the equivalent pinout tool built into STM32CubeIDE) before finalizing pin assignments, and do it before schematic capture, not during. Reason: unlike the SAM3X8E where most GPIO can serve almost any peripheral function fairly freely, STM32 peripherals are only available on specific pins via a fixed Alternate Function (AF0–AF15) table — SPI2_MISO, for example, is only broken out on a handful of specific package pins, not any GPIO. CubeMX (a) enforces that AF table automatically so you can't accidentally assign a peripheral to a pin that doesn't support it, (b) flags conflicts when two peripherals want the same pin, (c) configures the clock tree (critical here, since the 25MHz HSE → 480MHz core PLL → exact 48MHz USB clock chain in Section 11.2.1 has to be validated, not assumed), and (d) exports a pinout report you hand-transcribe into the KiCad schematic. It does not generate the KiCad schematic itself — that transcription step is manual — but doing the peripheral/pin planning in CubeMX first is the difference between a clean first-pass schematic and discovering a pin conflict after routing has started. Given that this document's Sections 5–7 have already decided which peripheral goes where functionally (SPI1 for TFT, SPI2 for module bus, TIM2/TIM5 for Q/R, etc.), the CubeMX pass itself should be quick — it's confirming/finalizing specific pin numbers against that already-settled functional plan, not re-deciding the plan from scratch. |
| --- | --- |

# 12. Glossary

| Term | Meaning |
| --- | --- |
| AF | Alternate Function — the STM32 mechanism mapping peripherals to specific GPIO pins (AF0–AF15) |
| CDC | Communications Device Class — the USB class that presents a virtual COM/serial port |
| CubeMX | STM32CubeMX — ST's graphical pin/clock/peripheral configuration tool |
| DAC | Digital-to-Analog Converter |
| DDS | Direct Digital Synthesis |
| DFU | Device Firmware Upgrade — a USB class / ST ROM-bootloader firmware-update mode |
| DRC | Design Rule Check (PCB) |
| ERC | Electrical Rule Check (schematic) |
| ETR | External Trigger — a hardware timer input on STM32 |
| FT | Five-volt-Tolerant (I/O pin designation) |
| GAACE_Core | GAA Custom Electronics' portable embedded core library set |
| HSE / LSE | High-Speed / Low-Speed External oscillator (main crystal / 32.768kHz RTC crystal) |
| LDAC | Load-DAC strobe — latches new values into a DAC output |
| lwIP | Lightweight IP — an open-source embedded TCP/IP stack |
| MSC | Mass Storage Class — the USB class that presents a device as a removable drive |
| NOR flash | A flash memory type suited to random read + simple filesystem use (vs. NAND) |
| QSPI | Quad Serial Peripheral Interface — high-speed serial interface to external flash |
| RMII | Reduced Media-Independent Interface — low-pin-count Ethernet MAC-to-PHY interface |
| SWD | Serial Wire Debug — ARM's 2-wire debug interface (subset of JTAG) |
| TWI | Two-Wire Interface — Atmel's term for I2C |
| VOS0 | Voltage Output Scaling level 0 — the STM32H7 power mode enabling max (480MHz) clock; unavailable on Rev Y silicon |